THe step rates of floppy drives (and other features) are controlled by

a set of tables in the DDBIOS ROM. Look at the listings for details.

THere are tables for the following drives at these locations:

8" sd @ F03F

8" DD @ F048

8" DD/256 byte sectors @ F051

5" SD @ F05A

5" DD @ F063

The table format is

DB	xx	;sectors per track

DB	xx	;tracks per side

DB	xx	;index header  gap for formatting

DB	xx	;gap 1

DB	xx	;gap 2

DB	xx	;gap 3

DB	xx	;restore command for floppy chip

DB	xx	;seek/no verify command

DB	xx	;seek/verify command

The last three commands have imbedded within them two bits that control

the step rate of the drive as follows:

bit 1   bit 0		8"drive		5"drive

0	0		3ms		6ms

0	1		6ms		12ms

1	0		10ms		20ms

1	1		15ms		30ms

Change these bits in the ROM (by burning another ROM!) to change the

drive step rates. The two versions of the PROMS on this disk have

the fastest and slowest step rates for the 8" drives within them.

####################################